MS50 magnetic scale

A sensor made using the principle of magnetic poles. The base bar is a uniformly magnetized steel strip. The S and N poles are evenly spaced on the steel belt, and the reading head reads the changes of the S and N poles to count. The magnetic scale is economical and efficient, and is suitable for long-distance measurement. Especially suitable for harsh environments such as oil, cutting chips, vibration, etc.

Technical Parameters

Material	High Performance Magnetic Stripe

width	10mm±0.1mm (can be customized)

thickness	1.3mm±0.1mm

thickness	Standard 1, 2, 5, 10 (customizable)

Accuracy grade (20°C)	±0.03mm

Operating temperature	-5～+75℃

Storage temperature	-5～+70℃

cover strip	Stainless steel + 3M double-sided tape

Strip Width	10mm±0.1mm

Strip Thickness	0.35mm (including double-sided tape)

single roll length	Standard roll 50m

degree of protection	IP67

选型说明

MS  XX - XXX

Magnetic grid length
(Example: 3m, magnetic strip length 3m)

50：Magnetic pole pitch 5mm              20: Magnetic pole pitch 2mm

MS: Magnetic scale

Advantages of magnetic scale

1. Non-contact measurement, no backlash problem

2. The magnetic scale is magnetically stable and has a long service life

3. High anti-pollution ability

4. The installation is simple and the allowable error is large

5. High cost performance

6. Can be cut with scissors